# Uniform Data Standard (UDS) v1.0 — Loudspeaker Companies & Cabinets

Goal: collect comparable, source-grounded data on every company’s speaker/cabinet designs, design philosophy, and case studies to fuel unorthodox, high-fidelity system design for 100–200 cap communal events.

## 0) Global rules

- Sources first: prefer manufacturer datasheets/whitepapers → patents → trusted press → 3rd-party measurements. Capture URL + date for each fact.

- Measured vs. claimed: every spec has provenance: manufacturer|independent|derived and confidence: high|med|low.

- Units/normalization: SI; frequency in Hz; SPL in dB re 20 µPa; sensitivity at 2.83 V/1 m (also store 1 W/1 m if supplied); power as AES/IEC if stated; angles in degrees (HxV).

- Nulls & estimates: use null for unknown, not “N/A”. Derived values flagged provenance: derived with method.

- Taxonomy: tag designs using the UDS Tech Vocabulary (Section 6) so cross-brand comparisons are queryable.

## 1) Company record (one per brand)

{

"company_id": "alcons_audio",

"name": "Alcons Audio",

"aka": [],

"founded_year": null,

"hq_country": "",

"key_people": [{"name":"","role":"","years":""}],

"ownership": "private|public|division_of:<parent>",

"lineage": {"predecessors": [], "spin_offs": []},

"philosophy": {

"tags": ["controlled_directivity","low_distortion","time_alignment"],

"summary": "2–5 sentence technical synopsis grounded in sources",

"notable_quotes": [{"quote":"","person":"","year":"","source":""}]

},

"supply_chain": {

"drivers_in_house": false,

"driver_vendors": ["B&C","BMS","PD","Faital","Eminence","SB Audience","Volt","Radian","ScanSpeak","Seas","Morel"],

"amp_dsp_partners": ["Powersoft","Lake","Hypex","ICEpower","Trinnov","Dirac","miniDSP"]

},

"standards_cert": ["THX","EN54-24","IECEx","ATEX","Dolby_Layout_Ready","AES67","Dante"],

"sources": [],

"audit_notes": ""

}

## 2) Model record (one per loudspeaker/cabinet)

{

"model_id": "brand_series_model_rev",

"company_id": "alcons_audio",

"name": "CRMS MkII",

"release_year": null,

"status": "current|discontinued|prototype",

"application": ["touring","theatre","cinema","home","studio","industrial"],

"form_factor": "point_source|line_array|column|horn|open_baffle|omni|sub|kick|coax_monitor|main_monitor",

"topology": {

"radiators": ["compression_1.4in","cone_12","cone_15","AMT","planar","pro_ribbon","coax_cd"],

"hf_loading": ["os_cd_horn","diffraction_slot","acoustic_lens","multi_entry_horn"],

"mf_loading": ["sealed_mid","cardioid_mid","mid_horn"],

"lf_alignment": ["sealed","reflex","passive_radiator","bandpass_4th","bandpass_6th","ppsl","tapped_horn","tl","paraflex","roar","front_loaded_horn","rear_loaded_horn","ripole"],

"directivity_methods": ["constant_q_horn","cardioid_enclosure","gradient_array","cbt_column","steerable_dsp","dipole","omni"],

"ports": ["flared_round","laminar_slot","multi_port","aperiodic_resistive","variable_length"],

"structure": ["birch","marine_ply","mdf_selective","aluminum","composite","cld_braced","3d_printed_waveguide"]

},

"drivers": [

{"position":"HF","make":"","model":"","exit_mm":35.6,"material":"Be|Ti|Polyimide|Ribbon|ProRibbon"},

{"position":"MF","make":"","model":"","size_in":10},

{"position":"LF","make":"","model":"","size_in":15,"count":2,"arrangement":"ppsl|iso|front"}

],

"key_specs": {

"bandwidth_hz": [null,null],

"f3_hz": null,

"f10_hz": null,

"sensitivity_db_2v83": null,

"sensitivity_db_1w": null,

"max_spl_db_cont": null,

"max_spl_db_peak": null,

"power_handling_w": {"aes": null, "program": null, "peak": null},

"impedance_ohm_nom": null,

"impedance_ohm_min": null,

"directivity_nominal": "90x40",

"directivity_map": [{"freq_hz":1000,"h_deg":90,"v_deg":40}],

"di_avg_db_500_10k": null,

"thd_percent_at_ref": {"freq_hz":1000,"spl_db":96,"value": null},

"group_delay_ms": [{"freq_hz":40,"ms":null},{"freq_hz":80,"ms":null}],

"dimensions_mm": {"w":null,"h":null,"d":null},

"weight_kg": null

},

"integration": {

"xover_points_hz": [null,null],

"recommended_processing": "FIR/IIR notes; cardioid filters",

"amp_requirements": {"ways":2,"per_way_w": [null,null],"min_headroom_db":6},

"connections": "NL4/NL8/plate_amp",

"mounting": ["pole","stack","fly_points","array_hardware"]

},

"variants": [{"name":"","differs_by":"coverage|finish|impedance|amp_dsp"}],

"measurements": {

"polars_available": true,

"cea2010_sub": [{"freq_hz":31.5,"db":null}],

"waterfall_etc": false,

"source_links": []

},

"case_studies": [

{

"site":"Club/Theatre/Cinema name",

"type":"club|theatre|cinema|outdoor",

"capacity": 180,

"geometry":"WxDxH m",

"deployment":"L/R + fills + subs",

"outcomes":"short bullet points (e.g., pattern control, SPL headroom)",

"lessons":"transferable design insights",

"sources":[]

}

],

"criticisms_limits": ["size for crossover target","port turbulence at 120 dB"],

"ip_patents": [{"id":"","year":null,"title":"","relevance":"horn|port|array|cardioid"}],

"r_and_d_flags": ["psychoacoustic_potential","infra_ready","transport_modular","cardioid_capable","synergy_candidate"],

"provenance": [{"field":"key_specs.max_spl_db_peak","source":"","provenance":"manufacturer","confidence":"med"}],

"last_audited": "YYYY-MM-DD"

}

## 3) Case-study record (reusable across models)

{

"case_id": "venue_yyyy_brand_model",

"venue": {"name":"","city":"","country":""},

"type": "club|theatre|cinema|festival|house_of_worship|industrial",

"capacity": null,

"room_dims_m": {"w":null,"d":null,"h":null},

"system_layout": {"mains":"L/R|LCR|array","fills":"front|out|delay","subs":"infra+kicks|cardioid"},

"coverage_target_deg": "HxV per zone",

"outcomes": ["SPL @ FOH","complaints reduced","mix translation"],

"measured_data": {"spl_dBA_slow":null,"spl_dBC_fast":null,"polars_collected":false},

"lessons": ["what to steal"],

"sources": []

}

## 4) Derived metrics (standard methods)

- Max SPL (approx.) if not provided:

 spl_max_est_db = sensitivity_db_2v83 + 10*log10(P_cont_w) + crest_db

 Store method:"derived", crest_db used (e.g., 3 dB for program, 6 dB for short peak).

- DI average: average DI over 500 Hz–10 kHz from polar map or manufacturer DI plot; store window.

- LF extension: compute F3/F10 by interpolation from magnitude data; store points and method.

- Group delay checkpoints: store at 40, 60, 80, 100 Hz for subs/kicks; at crossover for tops.

## 5) R&D relevance scoring (per model; weights fixed)

{

"scores": {

"hf_clarity_transient": 0.0,

"directivity_control_imaging": 0.0,

"sub_weight_infrasonic": 0.0,

"modularity_transport": 0.0,

"psychoacoustic_potential": 0.0,

"small_room_viability": 0.0

},

"weights": {

"hf_clarity_transient": 0.20,

"directivity_control_imaging": 0.20,

"sub_weight_infrasonic": 0.20,

"modularity_transport": 0.15,

"psychoacoustic_potential": 0.15,

"small_room_viability": 0.10

},

"composite": 0.00,

"rationale": {

"hf_clarity_transient": "driver/waveguide, HD data",

"directivity_control_imaging": "pattern stability evidence",

"sub_weight_infrasonic": "CEA-2010 / alignment",

"modularity_transport": "weight/dims/rigging",

"psychoacoustic_potential": "asymmetry/cardioid/OB tools",

"small_room_viability": "cardioid/dipole/low GD"

}

}

## 6) UDS Tech Vocabulary (controlled tags)

- Radiators: compression_1in|1_4in|2in|ring_radiator|coax_cd|cone|AMT|ribbon|planar|electrostatic|bmr|dml|pro_ribbon|piezo|ionic_plasma

- HF loading: os_cd_horn|exp_horn|tractrix|lecleach|diffraction_slot|acoustic_lens|multicell|paraline|multi_entry_horn|waveguide_large

- MF loading: sealed_mid|cardioid_mid|mid_horn|coax_mid_hf

- LF alignment: sealed|reflex|passive_radiator|bandpass_4th|bandpass_6th|bandpass_8th|ppsl|isobaric|tapped_horn|tl|mltl|paraflex|roar|front_loaded_horn|rear_loaded_horn|ripole|manifold

- Directivity: constant_q_horn|cardioid_enclosure|gradient_array|end_fire|cbt_column|steerable_dsp|dipole|omni|point_source_coherent

- Ports: flared_round|laminar_slot|multi_port|aperiodic_resistive|variable_length

- Structure: birch|marine_ply|mdf_selective|aluminum|steel|composite|cld_braced|honeycomb|3d_printed_waveguide

- Integration: passive_xover|active_plate_amp|external_amp_dsp|fir_linear_phase|all_pass_alignment|cardioid_filters

- R&D flags: infra_ready|cardioid_capable|psychoacoustic_potential|transport_modular|synergy_candidate|ob_hybrid_candidate

## 7) CSV extracts (for fast filtering)

companies.csv

company_id,name,founded_year,hq_country,ownership,philosophy_tags,drivers_in_house,amp_dsp_partners,standards_cert,last_audited

models.csv

model_id,company_id,name,status,application,form_factor,radiators,hf_loading,lf_alignment,directivity_methods,bandwidth_low,bandwidth_high,f3,f10,sens_2v83,max_spl_peak,directivity_nominal,di_avg_500_10k,weight_kg,w_mm,h_mm,d_mm,last_audited

cases.csv

case_id,model_id,venue,type,capacity,room_w,room_d,room_h,layout,subs_mode,measured_A,measured_C,outcomes,lessons,sources

## 8) Source record (attach anywhere)

{"title":"","url":"","publisher":"","date":"","type":"datasheet|whitepaper|press|review|measurement|patent","notes":"","confidence":"high|med|low"}

## 9) QA checklist (per model)

- All required fields present; units normalized; provenance/confidence set.

- Tech tags from Section 6 applied (≥1 in each applicable domain).

- At least 5 sources or an audit note explaining scarcity.

- Measured vs claimed clearly separated.

- Derived metrics flagged with method.

- Last-audited date set.

## 10) File conventions

- Markdown dossier: reports/<category>/<company>/<model>.md

- JSON sidecar: reports/<category>/<company>/<model>.json

- Indexes in reports/_indexes/*.csv

## 11) Minimal dossier template (Markdown)

# <Model> — <Company>

Use: <application> • Form factor: <form_factor> • Status: <status>

Topology: <radiators + loading + LF alignment + directivity>

Key specs: <bandwidth / F3 / sens @2.83V / max SPL / nominal pattern / weight>

Design notes: <2–4 bullets grounded in sources>

Case studies: <venue, year, takeaway>

Criticisms/limits: <bullets>

R&D flags: <list>

Sources: [1] … [2] …

Audit notes: <gaps, conflicts>

## 12) Ingestion protocol (agent)

- Resolve company_id; crawl official site for product index.

- Create one Model record per cabinet version (include variants).

- Parse PDFs for specs; extract polars/DI where available.

- Search patents with <company> + loudspeaker | horn | port | array | cardioid.

- Gather case studies from integrators/venues; attach lessons.

- Populate tech tags; compute derived metrics; score R&D relevance.

- Validate with QA checklist; write Markdown + JSON + CSV rows.