## 🌑 Node One Build Recommendation

A Hi-Fidelity Mythic Listening System (Portable 2.1 Edition)

### 🧊 1. Full-Range Tops (Stereo Clarity Portals)

#### 🪶 Function:

- Deliver extreme stereo imaging, transient clarity, and vocal fidelity across a wide range of textures (Burial to G Jones to DJ Screw)

#### 📐 Design:

- 2-way ported or sealed enclosure

  - 6.5”–8” high-sensitivity mid-woofer (Faital Pro 6FE200, SB Acoustics, or Dayton RS180)

  - 1” compression driver + waveguide (B&C DE10 or similar) or a high-quality soft dome tweeter for smoother top-end

- Enclosure: ~10–15 liters sealed or tuned ported with heavy bracing

#### 🔁 Future Upgrades:

- Add passive radiator or horn waveguide

- Replace tweeter with ribbon or AMT for more surgical detail

- Stackable horn-loaded mid-high expansion

### 🌊 2. Subwoofer (Infra Portal Root)

#### 🌋 Function:

- Deliver felt-not-heard bass from 25–35Hz with minimal distortion

- Keep bass tactile and warm for chopped & screwed, glitch, jungle

#### 📐 Design:

- Tapped Horn or Extended Reflex Enclosure

- Driver: 15”–18” high-Xmax woofer
 (Options: Dayton HO460, B&C 15TBX100, Eminence Lab 15)

- Cabinet design options:

  - THAM15 (tapped horn) for high-efficiency small-room use

  - Microwrecker or Tuba 24 for DIY infrasonic performance

  - Custom large reflex with ultra-low tuning

#### 🔁 Future Upgrades:

- Add a second sub for stereo or cardioid config

- Integrate sealed subs for ultra-tight low-end

### 🎛 3. Amplification & Crossover (Soul Splitter)

#### 💡 Starter Setup:

- Behringer NX1000D / NX3000D (Class D with DSP)

- Or: clean used stereo amp + MiniDSP 2x4 HD

#### 🧠 DSP Roles:

- Set crossover (~80–120Hz)

- EQ room response

- Limiters for driver safety

### 🔌 4. Input / Ritual Console (Signal Shrine)

- Rotary DJ mixer or minimalist mixer for vinyl / instrument integration

- Optional effects loop (reverb, delay, tape)

## 🧩 Modular Evolution Plan

| Phase | Additions |
| --- | --- |
| 🔹 Phase 1 | 2.1 system: Full-range tops + sub + DSP amp |
| 🔸 Phase 2 | Dedicated mid-high horn tops for expanded clarity |
| 🔺 Phase 3 | 2nd sub, spatial imaging tools (Ambisonics, surround) |
| 🌀 Phase 4 | Full mythopoetic enclosure design + reactive lighting or symbolic casing |

## 🔮 Symbolic Design Direction (Optional Guidance)

- Visual metaphor: Chrysalis of Sound
 → outer wood shell with geometric grillwork, inner light pulse behind the speaker cones

- Color palette: gunmetal, amethyst, deep violet, glitch white

- A name etched or sigil-burned into the wood for Node One

## 🛠 Estimated Cost (Starter Tier – DIY + Scavenged)

| Component | Est. Cost (USD) |
| --- | --- |
| Full-range drivers (pair) | $80–150 |
| Compression drivers/tweeters | $80–120 |
| Sub driver (1x 15"–18") | $100–180 |
| Wood & Materials | $80–120 |
| Amp (w/DSP or separate DSP) | $100–250 |
| Misc (cables, sealant, dampening) | $50 |
| Total | $500–800 DIY |